| Learning Outcome | At the end of this unit, participants will - Exposed to IC technology - Understand fundamental CMOS |
| --- | --- |

| Introduction | The ”Moore’s Law” (1965, Gordon Moore, co-founder of Intel): The maximum number of transistors on a chip doubles every 18 months. As a result the on-chip transistor count has increased one million times in three decades and has reached 30 million transistors (in microprocessors). |
| --- | --- |

| Overview on IC Technology | - First computer – Babbage Difference Engine (1832) - 25000 parts - Cost: ₤17,470 |
| --- | --- |

|  | - ENIAC ENIAC -- The first electronic computer (1946) The first electronic computer (1946) - Vacuum tube based. ~18000 tubes - 8,5ft height, 85ft long - First transistor - Designed in Bell Labs |
| --- | --- |

| First Integrated Circuit | - Bipolar logic - 1960’s (TTL, ECL) - Jack Kilby, Texas Instruments (1958), the monolithic integrated circuit, or microchip (patent #3,138,743), Nobel Prize in 2000 - Integrated Circuit history - Invention of BJT (1948) - First silicon transistor (1954) - MOS transistor (1960) - MOS integrated circuit (1962) - DRAM cell (1968) - Intel formed (1968), AMD formed (1969) - Microprocessor invented (1971) - 32-bit microprocessors (1980) |
| --- | --- |

| Intel 4004 Microprocessor | - 1972, 1000 transistors - 1 MHz operation, NMOS only - Pentium processor |
| --- | --- |
| Moore’s Law |  |

| IC Classification | - Circuit size (transistor count) - Circuit technology (BJT, BiCMOS, NMOS, CMOS. - Design style - standard cell - gate array - custom - Size classification (historical) - <100 SSI 1963 - 100-3000 MSI 1970 - 3000-30,000 LSI 1975 - 30,000-1,000000 VLSI 1980 - >1,000000 ULSI 1990 - >1 billion GSI 2010 |
| --- | --- |

| VLSI technology | - NMOS (N-type Metal-Oxide-Semiconductor) - Simple to design - CMOS (Complementary Metal-Oxide Semiconductor) - Less power consumption - East to design - BiCMOS (Bipolar CMOS) - Fast - Consume more power - ECL (Emitter-Coupled Logics) - Fast - Consume power - Less compact - GaAs (Gallium Arsenic) - Very fast - Hard to design |
| --- | --- |

| IC Technology today | - The ”Moore’s Law” (1965, Gordon Moore, co-founder of Intel): The maximum number of transistors on a chip doubles every 18 months. - As a result the on-chip transistor count has increased one million times in three decades and has reached 30 million transistors (in microprocessors). - Semiconductor memories (DRAM) reached the density of 1Gbits per chip. - The increase of the transistor count is the result of reduction in the size of the smallest manufacturable feature, say, the minimum path/line width. - The minimum feature size has decreased from around 10µm in the 1970s to less than 0.2µm in 2000. - In order to reduce power consumption the supply voltage has been reduced from 5V to around 2V. - It has been predicted that in 2009 we will have: - 200M transistors on a chip with the minimum feature size 100nm and the supply voltage 0.6–0.9V. The DRAM will have 64Gb per chip. - We will be working with AMI05 and TMS035 technologies with the minimum feature size being 0.5µm and 0.35µm, respectively. - Silicon (Si) is the most popular material use to build integrated circuits. Other possibilities include germanium (Ge) and gallium arsenide (GaAs). |
| --- | --- |

| MOS Transistor | - The flow of the current between the source and the drain is controlled by the electric field generated by the gate-substrate voltage. |
| --- | --- |

| NMOS | - Two n+ diffusion regions (’+’ indicates the high degree of doping) form the source and drain of the transistor. The area in between forms a conducting channel. Potentially, electrons, negative carriers, will form the current in the channel. - The gate, which is formed from a conductor, typically polysilicon, is insulated from the source-channel-drain structure (and from the substrate) by the layer of silicon dioxide. - The voltage between the gate and the substrate induces the electric field which controls the flow of the carriers in the channel. This gives the rise to the name: field-effect transistor (FET). - Transistor structure is completely symmetrical with respect to the source and drain. The role is defined by terminal voltages which establish the direction of the current (carriers) flow. |
| --- | --- |

| Cut off region for an NMOS Conducting NMOS | - We start with case when the gate voltage, VGS is smaller than the threshold voltage, VT . The drain voltage, VDS is in this situation irrelevant and can be zero. - The electric field induced by the gate voltage points down from the gate through the channel. This fields repels the majority carriers for the p-type substrate, that is, positive holes, from the channel hence forming a region depleted of carriers as shown in figure above. - As a result, due to the lack of free carriers, no current flows between the source and the drain at this stage, that is, ID = 0. - Source and drain are electrically connected. - If VDS>0V, then electron flow from source to drain provided VGS>VT - Electrons are the charge carriers for NMOS transistor. |
| --- | --- |

| Reference | - Mr. A.P.Papli´nski’s lecture notes - http://infopad.eecs.berkeley.edu/~icdesign/. |
| --- | --- |