| Learning Outcome | At the end of this unit, participants will - Know what hierarchy and instance arrays are. - Be able to traverse hierarchy using schematic and layout tools. - Be able to create and remove hierarchy in layout. |
| --- | --- |

| Introduction | For today design capacity with millions of transistor, it is almost impossible to carry out the implementation with a flat hierarchy design. In order to improve the design cycle throughput time, hierarchy concept is widely used. |
| --- | --- |

| Hierarchy Block diagram of hierarchy Layout representation of hierarchy Schematic and layout hierarchy | - Hierarchy - different levels of layout on a chip - Country ® state ® county ® city ® neighborhood - Chip ® cluster ® unit ® block ® cell - Instance - a cell placed within another cell - Perak is an instance of a state - Intel is an instance of a corporation - Child Cell - cell contained in larger cell is child of that cell - Parent Cell - cell containing smaller cells is parent of those cells - Hierarchy organizes & simplifies complexity - Hierarchy is usually determined by schematic design - Hierarchy should make sense for layout - Suggest modifications if necessary - Schematic organization of logic follows layout design - Schematic and Layout hierarchy should match - Simplifies use of design and verification tools: - Allows verification to be run at intermediate levels - Verification runtimes are faster - Easier to get parasitic data back into simulations - Easier to complete ECO (Engineering Change Order) |
| --- | --- |

| What is array? Stepped Cells Stepped Cells with space | - Array - ordered instantiation (placement) of same cell or objects in a design - Object can be a cell, group of cells, or other structure - Objects are related, physically and logically - Array can extend in X and/or Y direction - Flipping – each alternate instance is the mirror image in X and/or Y direction of the original cell - Stepping – the distance between neighboring instances - User can specify stepping (counted from origin to origin of instances) - Row vs column – row horizontal, column vertical - The example below shows multiple cell instances placed with the same orientation (no flipping) and same distance between neighbors. - The origin of the instantiated cell is determined by the small triangle in the lower left corner. - Origin at X:0, Y:0 of child cell - Example: 6x1 array of cell circuit - This example shows multiple cell instances placed with the same orientation, without flipping but with stepping. Notice the equal distance between each cell. - Example: 4x1 array of cell circuit |
| --- | --- |
| Flipped Cells Basic hierarchy planning Understand the constraints Basic Wire Planning | - This example shows multiple cell instances placed with the same orientation, with flipping and no stepping - Example: 5x1 array of cell circuit This is included - Understanding constraints - Basic Wire Planning - Block Sizing Fundamentals - Block Placement Fundamentals - Datapath Definition - Basic Datapath Pitch Planning - Estimating Layout Time - Interface signals - Critical signals - Metal allocations - Schedule When creating a wire plan, you need to: - Include all the input and output pins on your block. - Include all the M2/M3/M4 feed thru to your block. - Include shielding if needed. - Plan to have two or more power lines in larger-pitch cells. - Use the standard line pitch for signals and power lines (unless they are specified differently). - Plan to share tracks when possible. |

| Wire planning example | - Keep in mind, when parallel lines are routed at minimum design rule widths, there is no room to make a connection to the upper layer due to metal overlap requirements for the via. - Avoid making unnecessary jogs in routing (i.e. if a wire can go straight it should). This will help performance and area. - Your wire plan specifies the tracks needed for your over-the-cell routing, in conjunction with your port definitions. - You need a wire plan to see if your fub will be metal-limited and to reserve room for over-the-cell routing. Make sure this is planned before starting layout. - Estimate the minimum height required to draw this “buffcell” block. |
| --- | --- |

|  | Given: - cell pitch = 12um. - horizontal M2 lines (minimum space = 0.50um) . “in” @ metal width = 0.50um . “out” @ metal width = 0.80 um . Power lines @ metal width = 1.0um Then, device legged for 12 times. |
| --- | --- |

| Block sizing fundamentals Block placement fundamentals | The area required by a design is determined by the number of transistors “device-limited” or by the interconnects “metal-limited”. - If device-limited, the area is based on transistor density. The density is found by multiplying the number of transistors by the area needed for one transistor (in sq. mils/device). - If metal-limited, the cell’s area is determined by the total number of line pitch. - Place blocks efficiently using the least amount of routing. This is especially important for critical nets. - The Clock driver should be placed close to the clock input. - If available, use the “Layout Planner” as a template to start placing your blocks. - Utilize track-sharing and use the cross-reference file to understand how the blocks relate to each other. |
| --- | --- |

| Datapath Basic datapath pitch planning Estimating layout time | - Datapath describes layout that has the chips’ data buses running through it and being used in it, in a ‘bit-slice’ fashion. - Data busses are large (up to 86 bits for floating point), and typically use arrays to implement the logic; one cell is created, and then instantiated for each data bit. - Unique inputs and outputs to particular cell positions (bits) are usually handled with ports. - Making the right connections at the upper levels is called ‘programming’. - In planning a datapath pitch (cell width), the pitch will usually be determined by the widest cell. - It is desirable to match pitches through different parts of the chip to reuse common layout (for example, adders). - Pitches usually fall in the range between 25 and 60 microns. Outside these values, the aspect ratios become inefficient and/or cumbersome to use. - Datapath Pitch Planning Process: - Obtain a cross-reference file for all your blocks. - Have Wire planning and Cell pitch sizing ready. - Don’t skimp on power line widths. - If the pitch seems too narrow for an efficient layout, you can always upsize it. Don’t go any smaller than the worst-case pitch. - Be creative. - Plan the layout before estimating the schedule. - Determine your priorities: is getting the job done quickly or is density* more important for this project? - 75 devices/week is an example of productivity factor from Wmt project. The actual number depends on the type of layout (Datapath vs. Control vs. Ram) and the layout tools used. - Do not include your planning time here because Layout and Planning are two different tasks. You should have a plan ready before you can begin to estimate the layout time. |
| --- | --- |